# Template 4EB00 Q1 2026

# Special Topic: The Jet Engine

|  | Naam | Studentnr |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

?

Groep

| Name | Value | Unit |
| --- | --- | --- |
| Fuel | H2 |  |
| Tamb | 300.00 | [K] |
| P3overP2 | 9.00 | [-] |
| Pamb | 100000.00 | [Pa] |
| mfurate | 0.58 | [kg/s] |
| AF | 170.35 | [kg/kg] |
| v1 | 200.00 | [m/s] |

Assignment

Conditions

(Give your settings)

## Assignment Cycle Analysis of the Jet Engine based on NASA tables

Applying conservation equations and the NASA tables fill out table 1. The code and approach should be explained in detail in the following pages. As an example a worked-out part of the procedure for the diffusor (page 5) is given. You should still add the code snippet and provide the actual line numbers.

| State | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| P (kPa) |  |  |  |  |  |  |
| T (K) |  |  |  |  |  |  |
| v (m/s) |  |  |  |  |  |  |
| Table 1: Thermodynamic state summary |  |  |  |  |  |  |

## Diffusor

You do not have to fill this box. It will not be graded.

Code snippet diffusor [1-2] the one in the sheets. Don’t need yours here. Work starts at compressor stage

### Procedure (This just an Example. Your work starts at the compressor stage.)

Solved is first (line nr)

to determine T2 the isentropic relation can be used to solve for the pressure yielding

.

Which is implemented in (line nr). Mixture properties determined using f.i. (line nr)

The composition Yi is taken equal to that of normal air.

## Compressor

Code snippet Compressor [2-3]

### Procedure

## Combustor (2 code snippets)

Code snippet computation of composition before and after combustor [3-4]

Code snippet thermodynamic computations [3-4]

### Procedure

Complete table 2 (don’t forget to assign values to the red question marks)

|  | AF=? (equivalence ratio =? ) |  |
| --- | --- | --- |
| Mass fractions | Initial | Final |
| Fuel |  |  |
| O2 |  |  |
| N2 |  |  |
| CO2 |  |  |
| H2O |  |  |
| Rg [J/kg/K] |  |  |
| Table 2: Mixture composition before and after the combustor and the specific gas constants. |  |  |

## Turbine

Code snippet Turbine [4-5]

### Procedure

## Nozzle

Code snippet Nozzle [5-6]

### Procedure